THE BBC EMULATOR

version 0.3

copyright (C) 1996 by Tom Seddon

# Preamble

## Scope

This program emulates, or rather attempts to emulate, the behaviour of a BBC Microcomputer Model B with 32K of RAM, a paged ROM board with up to 16 paged ROMs and sideways RAM capability, and two disk drives. It does not support tapes, joysticks, serial or parallel port, speech, the ADC, the 1MHz bus, or anything else which involves plugging in hardware not mentioned above. It does emulate the 6845 and Video ULA chips, the System and User 6522 VIAs (with some exceptions) and the sound chip. This is all the hardware that is required to get most software working.

Please note that although the emulator does support sound, the support is currently rather buggy, not particularly accurate and rather incompatible (it works only with Soundblaster-compatible cards). Please read the section about sound before you try it.

## The BBC Micro… for those that don’t know

The BBC Micro was a very nice computer released in 1981 by Acorn Computers Ltd. It was designed to coincide with a computer literacy drive by the BBC (British Broadcasting Corporation) and there was a television series about the new-fangled home computers featuring it.

Despite what its critics (usually ex-Spectrum or ex-Commodore 64 owners!) would have you believe, the BBC was actually a very good computer. It offered a fast processor (2MHz 6502A), excellent expandability and superb firmware to go with the hardware. The operating system was full-featured (for the time) and easily expandable, the routine entry points were well-documented, and the computer came with an absolutely superb structured BASIC with an integrated 6502 assembler. Even compared to modern BASICs, BBC BASIC is lacking in only a few features, most importantly multi-line IF/THEN/ELSE and the WHILE/WEND construct. Pretty good for an 8-bit computer, and certainly better than the ultra-nasty Microsoft BASIC clones which came with most home computers in those days.

The BBC soon became popular, and although it never had the big-name support of the Atari 800, Commodore 64, Spectrum and (later) CPC, there was a steady supply of quality serious software, and a large selection of games. By 1985 and the end of the computer boom however Acorn was in dire financial straits, and was rescued by Olivetti; the BBC line was then discontinued, although another, enhanced BBC model (the 128K Master, released without the BBC tag) was released. With the advent of the 32-bit RISC Archimedes in 1987, the BBC was overshadowed rather. It steadily declined, and since about 1992/3 there has been virtually no software released for it. The BBC never had the same ‘hacker thing’ that the Commodore 64 – and to lesser extent the Spectrum – had, so today, apart from its use in some schools, it is pretty much entirely dead. This emulator is an attempt to keep alive the memory of this excellent and much underrated computer, and save it from unjustified obscurity.

# The Emulator

## Getting everything set up

Before you can do anything, there are a few things you will need. To actually get the emulated BBC running, you will need a copy of the BBC’s operating system ROM, and as well as this you will need one or more language/application paged ROM images. I recommend BASIC, although things such as Wordwise Plus will also work. For more details about this, and transferring BBC software to the PC, see the appendix.

Once you have these files, you will need to edit the BBC.CFG file to taste. The options therein are set to sensible defaults; for now you only need to worry about the OS= and ROM= lines. The first controls the location of the operating system ROM image; by default it is OS12.ROM in the current directory, but if you have put it somewhere else then set the line as appropriate.

The ROM= parameter specifies the location of a paged ROM. The first ROM= entry will be for DFFS.BIN; don’t worry about this, this is the filing system support ROM. The second entry points to BASIC2.ROM in the current directory; again, change the path if you’ve put it somewhere else or called it by a different name. (For more information about the format of the BBC.CFG file and the options contained therein, see the appendix.)

Once you’ve done all that, type BBC (and press return!) and sit back. Some messages will flash up the screen, and within about half a second you will be back at the good old BBC Computer 32K prompt. Isn’t it nice to be back at home...

If this didn’t happen – the emulator crashed with an error message, or the screen went into graphics mode but then stopped, etc. – please consult the appendix. Also check CWSDPMI.DOC to make sure you’re not doing or running something that isn’t supported.

## Points to note

There are several things which are different from a real BBC under the emulator, so it’s difficult to know exactly where to begin. The order of the following sections is fairly arbitrary.

### Keyboard

You may have noticed that the BBC keyboard is rather different from the PC. I’ve done my best to squeeze the Beeb’s rather idiosyncratic keyboard layout onto the PC’s, but it was a tricky job and you’ll need a 102-key UK-style keyboard. If you have a Windows 95 keyboard that would also be handy, since it makes the playing of some games (e.g. Thrust) a lot easier. (If you have a non-UK keyboard, please e-mail me with suggestions about how to lay out the keyboard. See the epilogue for details.)

In general I have tried to get a decent balance between the keycaps and the positioning. My only reference is the photograph on the cover of the first edition of Assembly Language Programming for the BBC Microcomputer by Ian Birnbaum (Macmillan, 1982), since I don’t have a BBC with me at university.

The alphanumeric keys have the same functions in the emulator as they do on the PC, although some of the shifted numeric keys have different effects (this is unavoidable). Apart from that the main key mappings are as follows:

` and ¬:	the BBC’s ^ and ~ key

- and _:		the BBC’s - and + key

= and +:		the BBC’s £ and _ key

# and ~:		the BBC’s @ key

; and : :	the BBC’s ; and + key

‘ and @: 	the BBC’s : and * key

The positioning of the * and ? keys should enable you to play most BBC games. A few, such as Bug Blaster and Jet Power Jack, have some really odd key combinations – hopefully the difficulty of using these with an emulator will condemn these games to the obscurity which I felt they always deserved.

The DELETE, CAPS LOCK, TAB and ESCAPE keys are mapped as you would expect. The cursor keys are mapped to both the standard cursor keys and the keypad cursors. The Insert key (both keypad and normal) acts as the COPY key, and the Num Lock key acts as SHIFT LOCK.

The positioning of the PC’s Caps Lock and Ctrl keys presents a problem for playing games (e.g. Thrust and Zalaga) which make use of these keys for left and right. For this reason, if you have a Windows 95 the left-hand Windows 95 key doubles as CAPS LOCK and the left-hand Alt key doubles as CTRL. This will allow you to play games such as this more easily.

The function key mapping is based on keycaps rather than positioning (F10 representing f0); I felt that for games such as Elite, Skirmish and the Supervisor from Exile it was the key number rather than its positioning that was more important. For some things though, such as Castle Quest, this doesn’t work so well; there will probably be an option in the next version to switch between these two modes.

Finally, the BREAK key is mapped to F12. I’m sorry about this, but the Pause/Break key involves detecting millions of bizarre scancodes and the F12 key is so much easier. I’ve had no complaints so far, but if you feel strongly on this issue please let me know.

There are also other hot-keys and key combinations you can press to do various things, but these are documented in their appropriate sections.

### Video

The video is definitely the big time-consumer in the emulator, so I’ve spent some time optimising it. I’ve cut some corners in terms of which video registers I emulate, and some timing issues, but these have not yet caused major problems.

The 6845 emulation is almost complete. The values in R1, R3, R4, R6, R7, R9, R12 and R13 are taken into account and affect the screen display. The emulator also uses the value in R5, although the screen can be moved down by no more than 10 scanlines. This is sufficient to allow the scrolling in Firetrack to work, but some games which move the screen up or down by a large amount for timing purposes (possibly Cyborg Warriors, but I’ve not done much research) get things wrong.

The Video ULA emulation is also virtually complete. The only problem is the undocumented ULA modes (10 characters per line and fast 6845 clock, and 80 characters per line and slow 6845 clock) – if these are selected the emulator will crash to DOS with an error message. I haven’t had a chance to try these values out on a real BBC, so I’m not sure what should be happening in these cases. Once I’ve had a play with a real BBC I’ll put some meaningful emulation there.

The only other point you should be aware of is an entirely cosmetic one. The screen update routines work by only updating those pixels which have changed since the last time the screen was refreshed, which makes the graphics emulation much faster. When hardware scrolling and changing the palette however the entire screen must be redrawn, since although the bytes in memory have not changed the screen’s appearance has. Thus, after these events the entire screen is redrawn.

Initially I had the screen updating immediately after palette and mode changes; I soon realised that this caused a problem since games that use a split-mode or split-palette display were causing the screen to be fully refreshed on every frame, since the palette or mode value was being changed on every frame. For this reason, the emulator now waits a certain number of refreshes before taking into account changes made to the ULA register or the palette, to ensure that full updates don’t take place too often. You can see this in effect if you go into mode 4 and type VDU 19,0,4;0; – the screen will take a while to react (usually about half a second or so).

This mainly affects games which use colour cycling for animation (Felix Meets The Evil Weevils, 3D Grand Prix, Overdrive) and games which make use of rapid flashing (Rocket Raid, Missile Base, Cylon Attack) – screens can look very odd. If you don’t like this, see the appendix for details on how to change the frequency of screen updates.

If you wish to force constant updates for any reason, simply hold down the Scroll Lock key. While this is held down, every frame will be drawn fully; unless you have a very fast computer, this will also slow down the emulator. Release it to return to normal update mode.

There is also the option to limit the emulator to updating the PC screen once every ‘x’ BBC frames – by default the PC screen is updated every time the BBC one would be, but you can have the PC updating at any rate, up to 1/25th of the speed of the BBC screen. If you have a slow video card you might like to play with this option to try and keep the speed up.

### Sound

Yuck. I’ve put sound support in this version, since I had someone asking when it would be done – well, here is what I have done so far. Currently it supports Soundblaster only; although it uses only the Adlib portion, it uses the Adlib ports through the Soundblaster (i.e. ports 2x8h and 2x9h rather than 388h), and also requires a BLASTER variable to find the Soundblaster base address.

Once you’ve got it working, be warned that it can sound rather odd. Some sounds are too high, and some are too low, and ENVELOPEd sounds never sound right (too quiet, or non-existent) and Speech! sounds pretty dreadful (but recognisable – just). Music often sounds very out-of-tune as well. I’d advise you to try the sound with each game you’d like, and see what it’s like.

If you’re looking for a good demonstration though I recommend Acornsoft’s Carnival, which sounds pretty much identical (bar the odd duff note) on the emulator. Zalaga isn’t too bad either, but the lasers sound rather off, and, apart from the fact that the laser noise continues after you stop firing (very annoying), Starship Command sounds OK as well.

### VIAs – User and System

The BBC has two 6522 VIAs, and they are both implemented. The System VIA features an addressable latch, and access to various hardware devices – of these only the keyboard and sound system are implemented. The joystick buttons and speech status (from port B) are all hardwired to the unpressed status and the ‘do-nothing’ status respectively.

The User VIA is rather useless in the emulator, although its timers, registers etc. are all implemented fully. On port A, the pin levels simply reflect the value written to them; on port B however the values on the pins fluctuate randomly (so reading from &FE60 gives random values). Some Acornsoft games, especially Snapper and Defender/Planetoid/Whatever seem to require this.

The main limitation in the VIA emulation is the fact that the timers are only updated once every 128 2MHz cycles (i.e. every 40 or so 6502 instructions); however, to solve this, when the VIA timer is read the number of cycles since its last update is subtracted from its value. Hopefully this will give the correct value, but I’ve not had a BBC since I implemented this feature so I’ve been unable to verify it. In theory games such as Hypersports and Yie-Ar Kung Fu, which decode themselves using VIA timer values, should work; however I don’t have Hypersports any more and my copy of Yie-Ar Kung Fu was hacked to death ages ago.

The other limitations in the VIA emulation are the shift register and the interrupt modes. The shift register would be very hard to emulate properly, so I’ve just left it out. Reads and writes to the shift register will change the value – but nothing will happen to it.

As for the interrupt modes, input interrupt modes are all implemented correctly (I hope) but the output ones are ignored since I don’t know what they’re for. I suspect this is what is causing many of the games to hang up, but since I have no idea whatsoever how to go about implementing these interrupt modes I haven’t experimented much.

# The Control Menu

At any time while using the emulator you can pop up a control menu by pressing the F11 key. After pressing F11 there will be a slight pause as the mouse routines initialise, and the BBC’s screen will then darken and the menu will pop up.

The control menu is as yet unfinished, but when I’ve implemented everything you’ll be able to change emulation options, swap around paged ROMs, check hardware status, change graphics mode and do other related things, all without having to quit and edit the BBC.CFG file. The exact format of the menu is not finalised either – although the dialog boxes are quite nice, something quick and easy – to program, that is – such as the menus used by Z80 are a possibility.

## The Working Options

I’d like to say that it’s purely coincidence that the first options to appear are the easiest... but I’d be lying...

### Save GIF screenshot

This replaces the Alt+PrtScr hotkey in the previous version. You can now save GIFs with any filename – there is a file selector with which you can choose the location and filename of the picture to save. GIFs are still 640x512, but you can always resize them. I’ll put in an option to make them a more sensible size next time.

### Power-on Reset

This is mainly designed for games which play with the *FX247 options (Holed Out! and W.A.R. among others) and hang the BBC when Break is pressed. This option is equivalent to power-cycling the Beeb – it erases the memory and resets the hardware totally, so you are must confirm before anything happens.

### About this program

I had to put this in first really :-)

### Continue

Continues where you left off.

### Exit to DOS

Quits you back to MS-DOS (or wherever you came from). Confirm your choice first! Note that the End+PgDn key combination still also works, but this might be gone in the next version.

## The Non-working Options

Just to give you an idea of what’s to come (hopefully!) in the next version.

### Load snapshot, Save snapshot

These will eventually load and save memory snapshots, similar to those so beloved of Spectrum emulator users. Unfortunately the standard BBC snapshot format has yet to be finalised, so I’ve left this option unwritten at the moment.

### System VIA Info, User VIA Info, 6845/Video Info

These will just give register dumps for the appropriate piece of hardware at the current moment. I’m currently debating whether to keep them in, since (because of the GIF snapshot facility) the menu can only be popped up at the end of each frame redraw – which makes these options fairly useless for debugging.

### Keyboard mapping

This will allow keyboard mapping, in case you have weird games which use odd keyboard combinations, or in case you don’t like what I’ve done.

### Emulation options

Anything which doesn’t fit into the other categories – stuff such as sound on/off, graphics update rate, accuracy refresh rate, that sort of thing.

### Graphics mode

Change the graphics mode. I’ve had this working, but Allegro refuses to draw the dialog box correctly – the buttons keep disappearing. When I’ve got to the bottom of all that, I’ll compile it with this option in.

# The Emulated Filing System

## Introduction

The emulator has built-in support for a file-based filing system through the use of illegal opcodes. The supplied DFFS (DOS File Filing System) ROM adds a wrapper round these functions, so to the user (and to programs) the system appears to be using a standard DFS interface.

To ensure that the DFFS is loaded, insert a line such as

ROM=DFFS.BIN

in your BBC.CFG file. It’s there with the supplied BBC.CFG so unless you’ve been doing some major changes you shouldn’t need this.

Make sure it’s in as high a ROM socket as possible, and also make sure there aren’t any other filing system ROMs in there; the DFFS will hopefully coexist with other filing systems, but it hasn’t been tested at all. As the emulator stands, other filing systems won’t work anyway so you don’t need them.

When you next load the emulator, in addition to the standard sign-on message you should also see Emulator DFFS. The DFFS is now available for use.

Please note: the DFFS is, at the moment, rather poorly implemented. Originally I was going to give it a complete overhaul for this release, but I haven’t had time. Please bear this in mind. As far as I can tell, there are no major bugs in the features I have written so far; if you find one, however, I’d be grateful if you would let me know.

## DFFS concepts and differences

The DFFS deals with disk files, which are basically several BBC files concatenated with a header at the front detailing load and execution addresses, etc. These files are not compatible with any other known disk format type (sorry!) but there are several programs provided to convert from various popular formats to DFFS format.

Apart from that, using the DFFS is very similar to using any Acorn-compatible DFS (except for the lack of * commands and random-access routines). Filenames consist of a 1-character directory and a 15-character filename (I just couldn’t resist increasing it!), and have 32-bit load and execution addresses and a 32-bit length. They also have attributes, but currently only the read-only attribute is taken into account.

The other major difference between most DFS systems and the DFFS is the value of PAGE. PAGE under the DFFS is set to &F00, rather than &1900, and provided that you do not use the *CAT command you can set PAGE to &E00 without any problems.

## Getting started with DFFS

There is a demo disk supplied, which has a couple of programs so you can check that the emulator works OK. Once you are at the sign-on prompt, you will need to mount the ‘disk’ into the ‘drive’. Type this:

*MOUNT DISK\DEMODISK 0

Note that DFFS files have no standard extension (they can be called anything), and that you must specify the path (if needed) on the command line in PC rather than BBC format. The last parameter is the drive number.

The disk will then be loaded in, and when the prompt reappears it has been mounted. Try cataloguing it to check. The header will look like this:

(no title)

Drive 0			Option 0: off

Directory :0.$		Library: ???

003/032 files

Note that the title will always be ‘(no title)’, since I haven’t got round to implementing titles yet. The drive, option, and directory indicators are the same as with the Acorn DFS. The Library directory is always ??? at the moment, since it isn’t taken into account. Finally, there is an indicator (never present in the Acorn DFS, annoyingly) to show you how many catalogue spaces are left on this disk.

Once you’ve finished playing, you must unmount the disk.

*UNMOUNT 0

Those are the main differences between the DFFS and DFS. Apart from that they are pretty much identical.

## DOS-based DFFS utilities

Because the disk format for the emulator is different from any other, I have written several conversion programs to enable you to use programs you might already have. In addition, there is also a utility to create new DFFS images. These are detailed here.

### WSCONV

This program converts a directory created with Wouter Scholten’s disk utility program into a single DFFS file. It takes one parameter – the name of the DFFS file to create – and scans through the directory looking for INF files. Once it finds one, it reads the data from that and adds the binary file related to the INF file into the catalogue, using the name supplied in the INF file. If there is no directory specified in the INF file, then $ is assumed.

### DCONV

This converts a DFS disk image into a DFFS file. There are two types of DFS image: single- and double-sided. Single-sided images are up to 200K in length, and double-sided images are up to 400K in length. Please note that DCONV, like BeebEm, expects double-sided images to have interleaved tracks.

This program takes three parameters – the name of the DFS image to convert, the name of the DFFS file to create, and a final parameter indicating what format the disk is. S indicates single-density, and D indicates double-density. If you specify the D parameter, it must be followed immediately by the side number you wish to convert – 0 for side 1 (drive 0 or drive 1), and 1 for side 2 (drive 2 or drive 3).

So, for example, if you were going to convert side 2 of the double-sided disk image STRYKER.IMG into the DFFS file STRYKER, you would use:

dconv stryker.img stryker d1

The program will handle 40- or 80-track images, and also images which have been truncated (i.e. their final unused sectors have been removed).

### DDOSCONV

Probably the least useful (for most!) of the three, this will convert Opus DDOS images into several DFFS files. You will first need some 80-track, double-sided, double-density DDOS images; these are easily created using Anadisk and are 720K in length. If you’re going to create them yourself, tell Anadisk not to save sector ID information and suchlike – the program will get confused if you do this.

The program takes just one parameter – the image to convert. It will read the image into memory, and for each volume (A, B, C etc) on each side it will create a single DFFS file. The filename of each new file will be the same as the main part of the filename of the original image, and the extension will be the side (not the drive) and volume number. So, for example, after converting a DDOS disk called TEST.IMG which had three volumes on each side, you would be left with:

TEST.0A, TEST.0B, TEST.0C, TEST.1A, TEST.1B, TEST.1C

with each DFFS file containing the files in the corresponding DDOS volume.

### MKIMG

This will create a new DFFS image. It takes two parameters – the name of the new file to create, and the number of catalogue spaces to be allowed. Once created, this new image can be *MOUNTed in the emulator and used as any other.

# Internet BBC resources

There’s not much that’s BBC-related on the Internet. The following are the best:

The BBC Lives!

http://www.idt.unit.no/~robert/bbc/bbc.html

An excellent site, this contains all the images and things from Wouter Scholten’s site along with plenty of BBC- and classic computer-related links.

Wouter Scholten’s BBC page

http://ww.cistron.nl/~wouters/bbc/bbc-etc.html

This page features a large collection of BBC software (mostly games) and various files related to the BBC. Any games you download from this page can be converted to DFFS format using the WSCONV utility.

I used to have a BBC page, but it’s no longer around (it will be back eventually though), and James Fidell, author of Xbeeb, used to have one as well but that has since moved. Hopefully his will re-appear, and with a bit of luck there may even be some more before the next version comes out.

# Epilogue

This is the first program I have ever released to the public at large, so I’d be grateful for any feedback. I’m looking in particular for the following:

·	Information about file formats – both disk formats, and any format you have used to store BBC programs on the PC. Writing new conversion programs has proved fairly straightforward so far, so if you have a request (and the information to go with it!) then I’ll have a go. I’ve got Watford 62-file information, so support for that will probably go in the next version;

·	Keyboard information – if you have a non-UK keyboard, please e-mail me with details of its layout and (if possible) suggestions for key mapping. I believe that most European keyboards are 102-key, so you should be OK with these, but US keyboards will probably have problems. I don’t have any foreign keyboards so please describe the keyboard layout exactly.

·	Suggestions for new features;

·	Game compatibility listings;

·	Problems with particular graphics cards, motherboards, BIOSes, memory managers, etc... and (if possible) how you solved them.

My e-mail address is:

T.W.Seddon@ncl.ac.uk

and you will always (until I leave/am thrown out of university) be able to download the most recent version of the emulator from:

http://www.ncl.ac.uk/~n5013784/bbc-emu.htm

In case you want to have a link to the ZIP file, that is:

http://www.ncl.ac.uk/~n5013784/bbc.zip

which will always be the latest version.

## Coming Not-so-Soon

Please watch out for version 0.4 of my emulator. It will probably be released after the summer, and, touch wood, should feature random-access filing support, many more DFS * commands and decent sound emulation. If I have the time, it may even feature a new, faster 6502 emulation core (written in assembler rather than C) but this isn’t definite – I still haven’t got the hang of AT&T assembler syntax yet.

## Getting the Source

If you want the source code, please send me an e-mail. Unless you live the UK I won’t even think about sending it to you over the post (it’s expensive! It’s a lot of hassle!), but if you live in the UK I should be able to manage. Please e-mail me first though. I’d also be interested in knowing why you want the source. (I’m not going to stop you doing anything with it, it’s just for my own interest!)

## Other Credits

The following people deserve to be thanked for their part, direct or indirect, in this program...

·	Sverre H Huseby (sverrehu@ifi.uio.no), who wrote the PD GIF routines used in the emulator. (His e-mail address is probably now out of date.)

·	DJ Delorie, for writing djgpp, and Charles Sandmann (sandmann@clio.rice.edu), for writing CWSDPMI to go with it.

·	Shawn Hargreaves (new e-mail address pending) for writing the Allegro graphics library;

·	Robert Schmidt, James Fidell and Wouter Scholten for their BBC-related WWW pages;

·	James Fidell, James Bonfield and David Gilbert for writing BBC emulators and releasing them with the source code;

·	Dan, for letting me use his computer for writing the emulator, when there were much better and more productive things to do such as Descent or Doom.

I think that’s everyone, although I’ve got that ‘there-should-be-one-more’ feeling. If you feel you’ve been missed out then e-mail me and I’ll put you there. (Unless you’ve done nothing and you just want a mention that is :-)

# Appendix A: Configuring the Emulator

The emulator is configured by means of a file called BBC.CFG. This is a standard ASCII file, and is a list of settings and values. The format of each line is:

<Setting>=<Value>

where <Setting> is the name of the setting you want to change, and <Value> is its new value. Comments, should you wish to put them in, must begin with a semicolon.

OS=<file>

This command specifies the file to use as the OS ROM image. It must be included.

ROM=(R:)(<file>)

This inserts a ROM into a ROM socket. The ROM= commands should be placed in order, with the ROM that needs to go into the highest priority socket (15) first, and the lowest priority socket last. If you want to keep a particular socket free, then leave the parameter blank (i.e. just have ROM=).

If you prefix the filename with an R: then that bank will be made writeable – that is, you will have a bank of sideways RAM. To create a blank bank of RAM, just use ROM=R:.

You must include at least one ROM= command.

AccurateRefreshFrequency=<Value>

This sets the delay between palette and mode changes and the screen updates. Normally this is set at 25 updates, but if this does not work very well with the programs you are using, set it to something different.

StartupMode=<Mode>

This sets the mode you wish the computer to start up in, and corresponds to keyboard links 6, 7 and 8. By default this value is 7, but (should you wish to) you can change it.

ShiftBreakAction=<0|1>

This corresponds to keyboard link 5, and sets the effect of a Shift+Break. Normally, BREAK on its own generates a reset, and Shift+Break generates a reset and autoboot; if ShiftBreakAction is set to 1 then this behaviour will be inverted. The default value is 0.

FastROMPaging=<0|1>

This changes the behaviour of ROM paging. Normally, when a blank ROM image is paged in the whole of the paged ROM area (16,384 bytes) is set to zero. If this setting is 1, then only the first 256 bytes will be set to zero; this is enough to convince the OS that the ROM area is now blank. If this causes problems – it hasn’t so far, at least not for me – then set it to 0.

The default value is 1.

MountImage=<drive>:<file>

This mounts a DFFS image into the specified drive before the emulator starts. You must specify the drive (before the colon) and the DFFS file to mount (after the colon).

If there are any errors when mounting the disk, the emulator will not start.

VideoMode=<0|1|2|3>

This sets the video mode the emulator runs in. The following are available:

0: 640x400 (compromise if you can’t use 640x350)

1: 640x480 (works with everything, but looks terrible)

2: 800x600 (looks best, but rather buggy)

3: 640x350 (looks good, but not all cards support this)

I generally use 640x350 – if you can’t get this resolution, try downloading UNIVBE (or whatever it is called at the moment) from http://www.scitechsoft.com/ which might help.

800x600 support is somewhat buggy – if you get strange effects (usually vertical lines at the bottom of the screen) then press Alt+PrtScr to remove them. This only works if the lines are in the area covered by the current screen mode – if you’re in Mode 7 it probably won’t work (I’ve not seen the lines extend far enough up).

ScreenUpdateFrequency=<Value>

Sets the number of BBC screen refreshes that should go by before the emulator updates the PC screen. Setting this to a value other than 1 will cause only every <Value>th screen to be displayed. If you have a slow graphics card or a slow computer you might wish to experiment with this value until you get something that will work well.

Any value below 5 will look OK and leave most games perfectly playable.

# Appendix B: ROMs

Before you can use the emulator, you will need copies on disk of at least two ROMs: the operating system ROM from the BBC, and a paged language ROM. The most useful paged ROM to have would be BASIC (either version) since many programs assume that this is installed, but a ROM such as Wordwise Plus or ADE would do.

The operating system is mapped into memory from &C000 to &FFFF inclusive. To save a copy on disk, use the following command:

*SAVE OS C000+4000

Saving a paged ROM is more tricky. All the ROMs installed in the BBC are mapped, one at a time, into the memory between &8000 and &BFFF inclusive. Using *SAVE will cause the filing system ROM to be mapped in, and a copy of this ROM will result (not useful). The following assembler/BASIC program will save a specified paged ROM to disk.

10PRINT “Please specify the paged ROM to save:”

20INPUT ROM%

30FOR T%=0 TO 2 STEP 2

40P%=&900:[opt t%

50.start

60lda &F4:pha

70lda #ROM%:sta &F4:sta &FE30

80lda #&00:sta &70:sta &72

90lda #&80:sta &71

100lda #&30:sta &73

110ldx #&40:ldy #0

120.copyloop

130lda (&70),Y:sta (&72),Y

140iny

150bne copyloop

160dex

170bne copyloop

180pla:sta &F4:sta &FE30

190rts

200]NEXT

210$&700=”SAVE ROM”+STR$(~ROM%)+” 3000+4000”

220X%=0:Y%=7:CALL &FFF7

230END

That was the easy bit. Now that you have the ROM images on the BBC end, you have to get them over to the PC.

# Appendix C: Transferring from the BBC to the PC

There are several methods of doing this.

## Serial Cable

This is by far the most depressing way of doing things. If you must do this, the best way is to get hold of an Xmodem program for the PC and the BBC, set up the serial parameters at both ends and get transferring.

Unfortunately, if you don’t have an Xmodem program for the BBC it is rather more tricky. The way I did it was to write a program under Qbasic on the PC which would receive ASCII serial data over the serial port and save it to disk. I then set the BBC to send text output through the serial port and did a straight *DUMP of the appropriate file and a *INFO at the end. I then wrote a program to decode the DUMP output and made a note of the execution and load addresses of the file.

I also wrote a program to automate this somewhat, and it is available from Robert Schmidt’s homepage. Unfortunately the data is saved in ASCII format, and although it’s fairly easy to convert you will have to write a program to do it. (It was only a very, very quick hack!)

## Reading BBC disks in a PC

This is the easier option, but there are a few caveats.

Since PCs have never used single-density disks, most – if not all – PC disk controllers do not have the capability to read single-density disks. This means that disks written to using a single-density DFS (Acorn DFS or any compatible) are unreadable on modern PCs. This also includes systems such as the Acorn 1770 DFS – although the 1770 is capable of reading and writing in double-density mode, the Acorn 1770 DFS uses it in single-density mode.

Double-density disks are fine however. If you have a double-density DFS (Challenger/Opus DDOS, Watford DDFS, Acorn ADFS, Solidisk DDFS, etc.) then disks from these should be readable. To read them, there are two programs available: Anadisk and FDCDEMO. Anadisk is available from SimTel mirrors (e.g. sunsite.doc.ic.ac.uk for those in the UK), as is FDCDEMO. Both are in the diskutil directory, I think – at sunsite, that’s

ftp://sunsite.doc.ic.ac.uk/packages/simtel-msdos/diskutil/

but don’t quote me!

To read the disks you will need to know the format of your BBC disks (40/80 track, 16/18 sectors per track). Enter this information into the program (see the documentation) and set it to read the disk into an image file. If possible, get it to save just the data, without sector ID information and other such stuff.

Once you have the disk images, you have the data on a PC disk but it is not in a usable format. I have written a utility (DDOSCONV – see above) which will convert Opus DDOS disks into a format usable by my emulator, and Wouter Scholten has written utilities (available from his homepage) which will extract Opus DDOS disks into several Acorn DFS images, but other formats are not yet supported.

## The Archimedes

If the above two options seem like a lot of hassle, or you have a disk format (most probably ADFS) that isn’t supported, you might like to try using an Archimedes. The Archimedes will read standard DFS disks (with the appropriate add-on program), but as-is it will read all ADFS disk types. It will also write MS-DOS disks, so you can copy the contents of an ADFS disk (directories and all) onto an MS-DOS disk and go from there. I’m not sure whether it will save load address and execution address information however, so this is probably most useful for text files and other such things, where these addresses aren’t so important. Any information much appreciated!

# Appendix C: Problems?

Are you having problems? Please go through this section, make sure you’ve done everything, and if that doesn’t work then please e-mail me. I try to reply to e-mails as quickly as possible, so if you haven’t received anything after about 4 or 5 days then I’ve obviously saved it in my ‘todo’ folder and forgotten about it. If this happens then please remind me!

I’m not sure what CWSDPMI’s compatibility is like. Here at university I havc easy access to only 2 computers (a 486dx2-100, and a P-90) although there are other university computers available. I’ve tested the emulator on all of them, and I’ve had no problem – although this is hardly comprehensive. If I have any problems during the rest of my testing then I will post a message on my home page and get to work on the problem.

## Video

The most probable difficulty is problems with your graphics card. The default graphics mode setting – 640x480x256 – should work with just about everything, but you never do know. If you get an ‘Allegro graphics initialisation error’ then the graphics library is having difficulties detecting your graphics card. The best way to solve this is probably to get your card to conform with the VESA interface – there may be a program that comes with your card, or you can try downloading UniVBE (or Scitech Display Doctor as I believe it’s now called) from

http://www.scitechsoftware.com/

Note that UniVBE is shareware and you have to register, although it does support linear framebuffer modes and a protected-mode interface for (allegedly) ultimate speed. If the registering aspect bothers you then there are VESA interface TSRs available for most graphics cards; if you have internet access they are probably downloadable from somewhere, and if you have NASCAR Racing or a similar SVGA-based game you may have been given a load of TSRs with that.

And as a final note – never assume your card is VESA-compatible because it says so on the box! Apparently many SVGA cards have buggy or incomplete VESA implementations, and some of those that claim to be VBE-compliant aren’t recognised.

## Sound

The most probable explanation for non-working sound is that your BLASTER environment variable has been set up wrongly. The correct format is:

BLASTER=A220 I5 D1 T4

with all numbers in hexadecimal. The number after A is the port, the number after I is the interrupt, the number after D is the DMA channel and the number after T is the soundcard type – 4 is standard Soundblaster, I think. The emulator only uses the port number, so ensure that this is set up correctly if nothing else.

If you have any other problems I’m afraid there’s not really much I can do. I’ll do some comprehensive testing at some point.

## General

The other possible problem is that the DOS extender, CWSDPMI, will have problems getting into protected mode. CWSDPMI.DOC, the original documentation, is supplied, but these two errors are the most probable: (with descriptions, from CWSDPMI.DOC)

"Error: Using XMS switched the CPU into V86 mode."

This message might be seen if you have your memory manager in AUTO mode. The only workaround in this case is to stop using AUTO mode.

"Protected mode not accessible."

This message should only be displayed if running CWSDPMI in a protected environment with no access to protected mode. In this case, DPMI should already be available and CWSDPMI would not be needed. I [Charles Sandmann <sandmann@clio.rice.edu>] would like to know if you see this message and DPMI is not available!

If you have any other problems with getting it to work, then please give me a mail. I can’t guarantee I can help, since the switch to protected mode and things like that are beyond my control; however I’ll give it a go.